| Ethernet VPN (EVPN) |  | December 19 2021 |
| --- | --- | --- |
| System/SW Functional Specifications document |

Proprietary Information

The information contained in this document is the sole property of Exaware LTD. The Disclosure of this information does not constitute the release of any proprietary rights therein. Permission to reproduce this information or parts disclosed herein must be obtained in writing from Exaware LTD.

Copyright © Exaware Ltd. All rights reserved. This document includes proprietary and confidential material of Compass Electro-Optical Systems Ltd. and may not be disclosed or used except with the express authorization of an authorized officer of Exaware Ltd.

Modification History

| Revision | Date | Name | Change Description |
| --- | --- | --- | --- |
| Rev 1.0 | 19-Dec-2021 | Indenbaum Michael | First draft |
|  | 12-Oct-2025 | Indenbaum Michael | Bug fixing |
|  | 30-Oct-2025 | Indenbaum Michael | The list of BGP common commands supported in the L2VPN EVPN context was added (chapter 2.2). |
|  | 11-Nov-2025 | Indenbaum Michael | Requirement IDs were added. |
|  | 06-Jan-2026 | Indenbaum Michael | Chapter “Debug CLI” was added. |

Disclaimer

No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose, without the express written permission of Exaware. Exaware retains the right to make changes to this document at any time, without notice. Exaware makes no warranty of any kind, expressed or implied, with regard to any information contained in this document, including, but not limited to, the implied warranties of merchantability or fitness for any particular purpose. Further, Exaware does not warrant the accuracy or completeness of the information, text, graphics, or other items contained within this document.

With respect to the products described herein, the user or recipient, in the absence of appropriate U.S. government authorization, agrees:

1) Not to re-export or release any such information consisting of technology, software or source code controlled for national security reasons by the U.S. Export Control Regulations ("EAR"), to a national of EAR Country Groups D:1 or E:2;

2) Not to export the direct product of such technology or such software, to EAR Country Groups D:1 or E:2, if such technology or software and direct products thereof are controlled for national security reasons by the EAR; and,

3) In the case of technology controlled for national security reasons under the EAR where the direct product of the technology is a complete plant or component of a plant, not to export to EAR Country Groups D:1 or E:2 the direct product of the plant or major component thereof, if such direct product is controlled for national security reasons by the EAR, or is subject to controls under the U.S. Munitions List ("USML").

At all times hereunder, the recipient of any such information agrees that they shall be deemed to have manually signed this document in connection with their receipt of any such information.

Table of Contents

1	Introduction	6

1.1	Scope	6

1.2	Overview	7

1.2.1	History	7

1.2.2	EVPN Concepts	7

1.2.2.1	EVPN Instance (EVI)	7

1.2.2.2	EVPN PE Models	7

1.2.2.3	Service Interface Types	9

1.2.2.4	Ethernet Segment (ES)	9

1.2.2.5	Ethernet Tag ID (ET ID)	9

1.2.2.6	Scenarios	10

1.2.2.7	EVPN BGP Routes and Extended Communities	11

1.2.2.8	EVPN Operation	12

1.3	Limitations	13

1.3.1	General Limitation	13

1.4	Glossary - Definitions, Acronyms, and Abbreviations	15

1.5	References	17

2	Requirements	18

2.1	Supported Standards (EVPNS-REQ#10)	18

2.2	BGP Common CLI in Context L2VPN EVPN (EVPNS-REQ#20)	18

2.3	VLAN Service Types	18

2.3.1	VLAN-Based Service Type (EVPNS-REQ#30)	18

2.3.1.1	VLAN-ID manipulation	18

2.3.1.2	Configuration Example	18

2.3.2	VLAN-Based Bundle Service Type Configuration (EVPNS-REQ#40)	20

2.3.2.1	VLAN-ID Validation	20

2.3.2.2	Configuration Example	20

2.3.3	VLAN-Aware Bundle Service Interface Type Configuration (EVPNS-REQ#50)	21

2.3.3.1	VLAN-ID Validation	21

2.3.3.2	Configuration Example	21

2.3.4	Port-Based Service Interface Type Configuration (EVPNS-REQ#60)	23

2.3.4.1	Configuration Example	23

2.4	Router Distinguisher (EVPNS-REQ#70)	24

2.5	Route Prioritization	24

2.6	MAC Learning	24

2.6.1	Static MAC Addresses (EVPNS-REQ#80)	24

2.6.2	Local MAC Learning (EVPNS-REQ#90)	24

2.6.3	Remote MAC Learning (EVPNS-REQ#100)	24

2.7	Multi-Homing	24

2.7.1	EVPN ESI and ES types (EVPNS-REQ#110)	25

2.7.2	Designated Forwarder Election (EVPNS-REQ#120)	26

2.7.2.1	Supported Algorithms	26

2.7.2.2	Algorithm Negotiation	26

2.7.3	Signaling Primary and Backup DF Elected PEs (EVPNS-REQ#130)	26

2.7.4	Aliasing Path (EVPNS-REQ#140)	26

2.7.5	Single-Active, All-Active and Load Balancing (EVPNS-REQ#150)	26

2.7.6	MAC Mobility (EVPNS-REQ#160)	27

2.7.7	Fast Convergence (EVPNS-REQ#170)	27

2.7.8	Split Horizon (EVPNS-REQ#180)	27

2.7.9	Interoperability with Single-Homing PEs (EVPNS-REQ#190)	27

2.8	Best Path Selection (EVPNS-REQ#200)	27

2.9	Forwarding	27

2.9.1	Forwarding Unicast Packets (EVPNS-REQ#210)	27

2.9.2	Multicast and Broadcast (EVPNS-REQ#220)	27

2.10	PMSI Tunnel attribute (EVPNS-REQ#230)	28

2.11	L2-Attr Extended Community (EVPNS-REQ#240)	28

2.12	ES-Import Route Target (EVPNS-REQ#250)	28

3	EVPN Routes	29

3.1	Ethernet A-D Per Ethernet Segment Route (1) (EVPNS-REQ#260)	30

3.2	Ethernet A-D Per EVPN Instance Route (1) (EVPNS-REQ#270)	31

3.3	MAC/IP Address Advertisement (2) (EVPNS-REQ#280)	31

3.4	Inclusive Multicast Ethernet Tag Route (3) (EVPNS-REQ#290)	32

3.5	Ethernet Segment Route (4) (EVPNS-REQ#300)	33

4	EVPN Label Types	35

4.1.1	LT1 (EVPNS-REQ#310)	36

4.1.2	LT2 (EVPNS-REQ#320)	36

4.1.3	LT3 (EVPNS-REQ#330)	37

4.1.4	LT4 (EVPNS-REQ#340)	37

5	EVPN Data Path	38

5.1	Forwarding Tables	38

5.1.1	MAC Unicast Forwarding Table (EVPNS-REQ#350)	38

5.1.2	BUM Split Horizon Table (EVPNS-REQ#360)	39

5.1.3	BUM Forwarding Table	40

5.1.3.1	LBUMT	40

5.1.3.2	RBUMT	40

5.2	Forwarding Rules (EVPNS-REQ#370)	41

5.2.1	General Forwarding Rules	41

5.2.2	Local Input Traffic	41

5.2.3	Remote Input Traffic	41

6	Configuration (EVPNS-REQ#380)	42

6.1	Interface Configuration	42

6.1.1	LACP Configuration	42

6.1.2	Interface and VID-ID Translation Configuration	42

6.1.3	Ethernet Segment Configuration	42

6.2	BGP Configuration	43

6.3	EVPN Configuration	43

7	Alarm and Syslog Messages	44

7.1	Alarms (EVPNS-REQ#390)	44

7.2	Syslog Messages (EVPNS-REQ#400)	44

8	Implementation	45

8.1	Development Milestones	45

8.2	EVPN Top Level Architecture	46

8.3	BCMRM MAC Learning support	47

8.3.1	MAC Address Type	48

8.3.2	Aging Time	48

8.3.3	MAC Learning	48

9	Debug CLI	50

9.1	BGP	50

9.2	EVPN	50

9.2.1	debug evpn events	50

10	Appendix	53

10.1	Appendix A – General & SW Abbreviations	53

List of Tables

Table 1: EVPN models	9

Table 2: EVPN Service Interface	9

Table 3: Terminology and Abbreviations	15

Table 4: References	17

Table 5: EVPN Routes	29

Table 6: EVPN MPLS Label Types	35

Table 7: VLAN-ID value	40

Table 8: General and SW Abbreviations	53

List of Figures

Figure 1: All-Active and Single-Active Load Balancing	11

Figure 2: EVPN Operations	13

Figure 3: EVPN Top Level Architecture	46

Figure 4: BCMRM MAC Learning Top Level Architecture	47

# Introduction

## Scope

The document specifies a new Layer 2 service called Ethernet VPN EVPN).

## Overview

### History

For scenarios requiring Ethernet multipoint connectivity, MPLS deployments leverage Virtual Private LAN Services (VPLS) [RFC4761, RFC4762]. A VPLS service is built with a full-mesh of Ethernet pseudowires among MPLS Provider Edge (PE) routers that are part of a given layer-2 broadcast domain. A VPLS PE emulates an Ethernet bridge by performing data-plane MAC learning (also known as “flood and learn” operation) against traffic arriving over the pseudowires and local interfaces. Existing VPLS PE does support only single-home mode.

The new Layer 2 service Ethernet VPN (EVPN) [RFC7209], [RFC7432bis] is a next generation solution that provides Ethernet multipoint services over MPLS networks. An EVPN comprises of customer edge (CE) devices that are connected to provider edge (PE) devices, which form the edge of the MPLS infrastructure. A CE device can be a host, a router, or a switch. The PE devices provide Layer 2 virtual bridge connectivity between the CE devices. A CE device can be connected to a few PE devices. There can be multiple EVPNs in the provider network. Learning between the PE routers occurs in the control plane using BGP, unlike traditional bridging, where learning occurs in the data plane. Additionally, to transparent bridging EVPN supports VLAN-Aware bridging.

### EVPN Concepts

#### EVPN Instance (EVI)

An EVI represents a VPN on a PE router. It serves the same role of an IP VPN Routing and Forwarding (VRF), and EVIs are assigned import / export Route Targets (RTs). Depending on the service multiplexing behaviors at the User to Network Interface (UNI), all traffic on a port (all-to-one bundling), or traffic on a VLAN (one-to-one mapping), or traffic on a list / range of VLANs (selective bundling) can be mapped to a Bridge Domain (BD). This BD is then associated to an EVI for forwarding towards the MPLS core.

#### EVPN PE Models

The PE EVPN Instance ([RFC7432bis], chapter 6.4) includes the following:

- MACT Forwarding Table (MACT).

- One or more Broadcast Domains (BDs). A BD is identified by VLAN-ID.

- RD

- RT

We can differ the following EVPN models:

Table 1: EVPN models

| Model | RFC7432bis | This Document |
| --- | --- | --- |
| A | One BT and one BD. | MACT with key MAC only (VLAN-ID = 0) and one BD |
| B | A few pairs of <BT, BD> | MACT with key <non-zero VLAN-ID, MAC> and a few BDs |

#### Service Interface Types

The standard ([RFC7432bis]), chapters 6.1 – 6.3.1)) defines a few Service Interface types. The types specify customer service provided by PEs.

The following service types must be supported (see the CLI command service-type):

Table 2: EVPN Service Interface

| Model | RFC7432bis Service Type Name | CLI Service Type Name |
| --- | --- | --- |
| A | VLAN-Based | vlan-based |
| A | VLAN Bundle | vlan-bundle |
| A | Port-Based | port-based |
| B | VLAN-Aware Bundle | vlan-aware-bundle |

#### Ethernet Segment (ES)

Ethernet Segments are used to represent “site” connections. They are associated with the access-facing interfaces (physical or logical (LAG, sub-interface)) that they represent and are assigned a unique identifier (referred to as Ethernet Segment Identifier (ESI)). Furthermore, a site can be connected to one or more PEs attached to a common MPLS core to provide access redundancy. With this, the solutions can support PE geo-redundancy and multi-homing (beyond dual homing) requirements. Lastly, a site can be used to connect to a single device or to an access network.

#### Ethernet Tag ID (ET ID)

An Ethernet Tag ID is a 32-bit field containing either a 12-bit or 24-bit identifier that identifies a particular broadcast domain (e.g., a VLAN) in an EVPN instance.  The 12-bit identifier is called    the VLAN ID (VID).  An EVPN instance consists of one or more broadcast domains (one or more VLANs).  VLANs are assigned to a given EVPN instance by the provider of the EVPN service.  A given VLAN can itself be represented by multiple VIDs.  In such cases, the PEs participating in that VLAN for a given EVPN instance are responsible for performing VLAN ID translation to/from locally attached CE devices.

|  | Note: This document attempts to use VLAN-ID instead of ET-ID. |
| --- | --- |

#### Scenarios

The following scenarios are available:

- Single-Homed Device (SHD)

- Multi-Homed Device (MHD)

For MHD EVPN defines two load balancing modes of operation:

- All-active load balancing: Supports multi-homed devices with per-flow load balancing. This mode allows the access device to connect via a “single” Ethernet bundle to multiple PEs and to send / receive traffic of the “same” VLAN from all the PEs in the same Ethernet segment.

- Single-active load balancing: Supports multi-homed devices with per-vlan load balancing. In this mode, the access device connects via “separate” Ethernet bundles to multiple PEs. PE routers in turn automatically perform service carving in order to divide VLAN forwarding responsibilities across the PEs in the Ethernet segment. The access device learns via the data-plane which Ethernet bundle to use for a given VLAN.

Figure 1: All-Active and Single-Active Load Balancing

#### EVPN BGP Routes and Extended Communities

EVPN PEs signal and learn MAC addresses over the core via BGP. For this purpose, a new address-family in Multi-Protocol BGP (MP-BGP) and new BGP extended communities allow PE routers to advertise and learn prefixes that identify MAC addresses and Ethernet segments over the network. This contrasts with existing VPLS solutions, which rely solely on data-plane learning. Control-plane learning brings a number of benefits that allow EVPN to address the VPLS shortcomings listed previously, including support for multi-homing with per-flow load balancing. Finally, by using BGP as a common VPN control-plane, providers can now leverage their operational experience and scalability characteristics inherent to IP VPNs, for their Ethernet offerings.

A total of four new BGP routes have been defined in order to collectively achieve the following functions:

- MAC address reachability

- MAC mass withdrawal

- Access split-horizon filtering

- Aliasing

- VPN endpoint discovery

- Redundancy Group discovery

- Designated forwarder election

When used in conjunction with an MPLS data-plane, these BGP routes also signal the MPLS labels associated with MAC addresses and Ethernet segments. This separates the new technologies from existing L2VPN solutions. EVPN no longer require the signaling and maintenance of pseudowires. EVPN MPLS labels represent Multipoint-to-Point (MP2P) Label Switched Paths (LSPs). Similar to IP VPNs, an EVPN MPLS label is signaled for use by the rest of the PEs in the VPN. There is no longer a requirement to signal separate point-to-point pseudowire Virtual Circuit (VC) labels for each remote PE. This brings the scale of L2VPNs to the same level as IP VPNs.

#### EVPN Operation

At startup, PEs exchange EVPN routes to advertise the following:

- Ethernet segment reachability: The PE discovers remote ESes and their corresponding redundancy mode (all-active or single-active). In case of segment failures, PEs withdraw the routes used at this stage in order to trigger fast convergence by signaling a MAC mass withdrawal on remote PEs.

- Redundancy Group membership: PEs connected to the same Ethernet segment (multi-homing) automatically discover each other and elect a Designated Forwarder (DF) that is responsible for forwarding Broadcast, Unknown unicast and Multicast (BUM) traffic for a given EVI.

- VPN membership: The PE discovers all remote PE members of a given EVI. In the case of a multicast ingress replication model, this information is used to build the PE’s flood list associated with an EVI.

After the initial EVPN route exchange, and when customer traffic arrives at the PE, EVPN MAC advertisement routes distribute reachability information over the core for each customer MAC address learned on local Ethernet segments. Each EVPN MAC route announces the customer MAC address, and the Ethernet segment associated with the port where the MAC was learned from, as well as an MPLS label. This EVPN MPLS label will be used later by remote PEs when sending traffic destined to the advertised MAC address.

Figure 2: EVPN Operations

## Limitations

### General Limitation

This specification includes the following limitations:

- The Layer 3 VPN is not supported:

- The following fields into the created MAC/IP Advertisement Routes are filled by 0 and are ignored in the ones received ([RFC7432bis], chapter 7.2):

- IP Address Length

- MPLS Label2

- Only Ingress Replication is supported, P2MP LSPs are not supported ([RFC7432bis], chapter 16).

- Only Data-Plane MAC learning by software on interfaces attached to local CEs ([RFC7432bis, 14.2.1) is supported. Optional functionality described in ([RFC7432bis], chapter 10) is not supported.

- The "Unique VLAN EVPN" scenario as a special case ([RFC7432bis], chapters 4, 7.9, and 7.10.1) is not supported.

- Optional functionality described by ([RFC7432bis], chapters 10) is not supported.

- Only ESI types 0, 1 and 4 are supported.

- Only the following DF selection algorithms (see the CLI command service-carving) are supported:

- default

- highest-random-weight

## Glossary - Definitions, Acronyms, and Abbreviations

Table 3: Terminology and Abbreviations

| AC | Attachment Circuit. |
| --- | --- |
| All-Active Redundancy Mode | When all PEs attached to an Ethernet segment are allowed to forward known unicast traffic to/from that Ethernet segment for a given VLAN, then the Ethernet segment is defined to be operating in All-Active redundancy mode. |
| BFD | Backup-Designated Forwarder. |
| BD | Broadcast Domain. In a bridged network, the broadcast domain corresponds to a Virtual LAN (VLAN), where a VLAN is typically represented by a single VLAN ID (VID) but can be represented by several VIDs where Shared VLAN Learning (SVL) is used per [IEEE.802.1Q_2014]. |
| BT | Bridge Table. An instantiation of a Bridge Forwarding Table on a MAC-VRF. |
| BUM | Broadcast, unknown unicast, and multicast. |
| CE | Customer Edge device. A CE device can be host, switch, or router. |
| DF | Designated Forwarder. |
| ES | Ethernet Segment. When a customer site (device or network) is connected to one or more PEs via a set of Ethernet links, then that set of links is referred to as an 'Ethernet segment'. |
| ESI | Ethernet Segment Identifier. A unique non-zero identifier that identifies an Ethernet segment is called an 'Ethernet Segment Identifier'. |
| ET | Ethernet Tag. Used to represent a BD that is configured on a given ES for the purposes of DF election and <EVI, BD> identification for frames received from the CE. Note that any of the following may be used to represent a BD: VIDs (including Q-in-Q tags), configured IDs, VNIs (Virtual Extensible Local Area Network (VXLAN) Network Identifiers), normalized VIDs, I-SIDs (Service Instance Identifiers), etc., as long as the representation of the BDs is configured consistently across the multihomed PEs attached to that ES. |
| ET ID | Ethernet Tag Identifier. Normalized network wide ID that is used to identify a BD within an EVI and carried in EVPN routes. |
| EVI | An EVPN instance spanning the Provider Edge (PE) devices participating in that EVPN. An EVI may be comprised of one BD (VLAN-based or VLAN Bundle services) or multiple BDs (VLAN-aware Bundle service). |
| Fast Convergence | A mechanism to efficiently and quickly signal, to remote PE nodes, the need to update their forwarding tables upon the occurrence of a failure in connectivity to an Ethernet segment. |
| MACT | An EVPN Instance Unicast Forwarding Table. |
| MHD | Multi-Homed Device. |
| MP2MP | Multipoint to Multipoint. |
| MP2P | Multipoint to Point. |
| Non-DF | Non-Designated Forwarder. |
| Normalized VLAN-ID | VLAN-ID after VLAN-ID translation (passed via MPLS). |
| P2MP | Point to Multipoint. |
| P2P | Point to Point. |
| P-tunnel | A tunnel through the network of one or more SPs. In this document, P-tunnels are instantiated as bidirectional multicast distribution trees. |
| PE | Provider Edge device. |
| Single-Active Redundancy Mode | When only a single PE, among all the PEs attached to an Ethernet segment, is allowed to forward traffic to/from that Ethernet segment for a given VLAN, then the Ethernet segment is defined to be operating in Single-Active redundancy mode. |
| SHD | Single-Homed Device. |
| VLAN-ID | VLAN Identifier |

|  | Note: This document attempts to use MACT instead of BT of [RFC7432bis]. The BT table has one key – MAC Address. The MACT table has two keys: VLAN-ID and MAC address. |
| --- | --- |

## References

Table 4: References

| Index | Reference description and title |
| --- | --- |
| CLI | EVPN CLI, https://exaware.atlassian.net/wiki/spaces/RNDDocs/pages/2729967663/EVPN |
| Pref-DF | RFC 9785 Preference-Based EVPN Designated Forwarder (DF) Election, June 2025 |
| RFC4364 | RFC 4364 BGP/MPLS IP Virtual Private Networks (VPNs), February 2006 |
| RFC4761 | RFC 4761 Virtual Private LAN Service (VPLS) Using BGP for Auto-Discovery and Signaling, January 2007 |
| RFC4762 | RFC 4762 Virtual Private LAN Service (VPLS) Using Label Distribution Protocol (LDP) Signaling, January 2007 |
| RFC6514 | RFC 6514 BGP Encodings and Procedures for Multicast in MPLS/BGP IP VPNs, February 2012 |
| RFC7209 | RFC 7209 Requirements for Ethernet VPN (EVPN), May 2014 |
| RFC7432 | RFC 7432 BGP MPLS-Based Ethernet VPN, February 2015 |
| RFC7432bis | BGP MPLS-Based Ethernet VPN draft-ietf-bess-rfc7432bis-13 |
| RFC8340 | RFC 8340 YANG Tree Diagrams, March 2018 |
| RFC8584 | RFC 8584 Framework for Ethernet VPN Designated Forwarder Election Extensibility, April 2019 |
| Phases | EVPH Phases 01, https://exaware.atlassian.net/wiki/spaces/RNDDocs/pages/2729967663/EVPN |

# Requirements

## Supported Standards (EVPNS-REQ#10)

The implementation must support [RFC7432bis] with changes specified by this document.

## BGP Common CLI in Context L2VPN EVPN (EVPNS-REQ#20)

The following CLI commands must be supported in the L2VPN EVPN context:

- allow-as-in

- capability

- inbound-soft-reconfiguration

- maximum-prefix

- private-as

- route-reflector-client

- weight

- group

## VLAN Service Types

### VLAN-Based Service Type (EVPNS-REQ#30)

#### VLAN-ID manipulation

The system must replace VLAN-ID in an input frame if it is needed to match to the configured VLAN-ID.

#### Configuration Example

The following example shows how to configure two EVPN instances of the vlan-based type with normalized VLAN-IDs 4 and 6 on Ethernet ports x-eth 0/0/1, x-eth 0/0/2, x-eth 0/0/3 and LAGs agg-eth 1 and agg-eth 2:

config

interface agg-eth 1

l2-transport enable

interface agg-eth 2

l2-transport enable

interface x-eth 0/0/1

l2-transport enable

interface x-eth 0/0/2

l2-transport enable

interface x-eth 0/0/3

l2-transport enable

!

interface x-eth 0/0/1.4

l2-transport enable

interface x-eth 0/0/2.4

l2-transport enable

interface x-eth 0/0/3.5

l2-transport enable

interface x-eth 0/0/1.6

l2-transport enable

interface x-eth 0/0/2.6

l2-transport enable

interface x-eth 0/0/3.6

l2-transport enable

!

l2-services

evpn evpn1

service-type vlan-based

interface agg-eth 1.4

interface agg-eth 2.4

interface x-eth 0/0/1.4

interface x-eth 0/0/2.4

interface x-eth 0/0/3.5

evpn evpn2

service-type vlan-based

interface agg-eth 1.6

interface agg-eth 2.6

interface x-eth 0/0/1.6

interface x-eth 0/0/2.6

interface x-eth 0/0/3.6

### VLAN-Based Bundle Service Type Configuration (EVPNS-REQ#40)

#### VLAN-ID Validation

An input frame with VLAN-ID not belonging to configured bundle must be dropped.

#### Configuration Example

The following example shows how to configure an EVPN instance of the EVPN vlan-bundle type on Ethernet port x-eth 0/0/1 and LAG agg-lag 1:

config

interface agg-eth 1

l2-transport enable

interface x-eth 0/0/1

l2-transport enable

!

interface agg-eth 1.1

l2-transport enable

vlan-id 1-4

interface x-eth-0/0/1.1)

l2-transport enable

vlan-id 1-4

!

l2-services

evpn evpn1

service-type vlan-bundle

interface agg-eth 1.1

interface x-eth 0/0/1.1

### VLAN-Aware Bundle Service Interface Type Configuration (EVPNS-REQ#50)

#### VLAN-ID Validation

An input BUM frame with VLAN-ID not belonging to configured bundle must be dropped.

#### Configuration Example

Example 1. The following example shows how to configure one EVPN instance of the EVPN vlan-aware-bundle type on Ethernet port x-eth 0/0/1 and LAG agg-lag 1 for all VLAN-IDs:

config

interface agg-eth 1

l2-transport enable

interface x-eth 0/0/1

l2-transport enable

!

interface agg-eth 1.1

l2-transport enable

vlan-id 1-4094

interface x-eth 0/0/1.1

l2-transport enable

vlan-id 1-4094

!

l2-services

evpn evpn1

service-type vlan-aware-bundle

interface agg-eth 1.1

interface x-eth 0/0/1.1

Example 2. The following example shows how to configure three EVPN instances on one Ethernet port x-eth 0/0/1:

- VLAN-Aware Bundle EVPN evpn1 for VLAN-IDs 1-3

- VLAN-Aware Bundle EVPN evpn2 for VLAN-IDs 4-5

config

interface x-eth 0/0/1

l2-transport enable

!

interface x-eth 0/0/1.1

l2-transport enable

vlan-id 1-3

interface x-eth 0/0/1.2

l2-transport enable

vlan-id 4-5

interface x-eth 0/0/1.3

l2-transport enable

vlan-id 10

!

l2-services

evpn evpn1

service-type vlan-aware-bundle

interface x-eth 0/0/1.1

evpn evpn2

service-type vlan-aware-bundle

interface x-eth 0/0/1.2

evpn evpn3

service-type vlan-aware-bundle

interface x-eth 0/0/1.3

### Port-Based Service Interface Type Configuration (EVPNS-REQ#60)

#### Configuration Example

The following example shows how to configure an EVPN instance of the EVPN port-based type on LAG agg-lag 1 and Ethernet port x-eth 0/0/1:

config

interface agg-eth 1

l2-transport enable

interface x-eth 0/0/1

l2-transport enable

!

l2-services

evpn evpn1

service-type port-based

interface agg-eth 1

interface x-eth 0/0/1

## Router Distinguisher (EVPNS-REQ#70)

Following [RFC7432bis], chapter 7.9, the system must automatically generate the following RD for an EVPN instance:

- Type Field: 1

- Administrator subfield: the IPv4 address configured on loopback 0.

- Assigned Number subfield: the unique EVPN Instance number generated by the system.

## Route Prioritization

The implementation must support the Route Prioritization specified in ([RFC7432bis], chapter 7.12).

## MAC Learning

### Static MAC Addresses (EVPNS-REQ#80)

Configuration and advertising of local MAC addresses MUST be supported.

### Local MAC Learning (EVPNS-REQ#90)

The implementation must support Data-Plane MAC learning by software on interfaces attached to local CEs ([RFC7432bis, 14.2.1). The Control-Plane learning on these interfaces ([RFC7432bis, 14.2.2) is not supported.

MAC learning from the LACP messages is not permitted.

### Remote MAC Learning (EVPNS-REQ#100)

The Remote MAC learning (see [RFC7432bis], chapter 9.2) MUST be supported.

## Multi-Homing

A Multi-Homed Ethernet Segment ([RFC7432bis), chapter 5)) can be configurated on a LAG interface. The LAG must be a LACP LAG and the same LACP System MAC address and LACP Administrative Key must be configured in all PEs connected to this Ethernet Segment (see CLI commands lacp-system-mac and lacp-key).

Additionally, an ES can be configured on a Single-Homed interface (physical ethernet port or LAG). For this ES:

- The router does not advertise the Ethernet Segment Route (4).

- The router advertises this ES as single-active in the Ethernet A-D Per Ethernet Segment Route (1).

- The router advertises itself as the Designated Forwarder in the Ethernet A-D Per EVPN Instance Route (1).

- The router fixes misconfiguration if this ES is advertised by another router.

### EVPN ESI and ES types (EVPNS-REQ#110)

The following ESI types must be supported (see CLI commands identifier):

- Type 0

- Type 1 (LACP LAGs only)

- Type 4 (single-home interfaces only)

The ESI type specifies whether the ES is single home or multi-homing ethernet interface:

| ESI Type | ES TYpe |
| --- | --- |
| 0 | Multi-Homed |
| 1 | Multi-Homed |
| 4 | Single home |

### Designated Forwarder Election (EVPNS-REQ#120)

#### Supported Algorithms

The implementation must support the following algorithms of Designated Forwarder Election (see command service-carving):

- Default ([RFC7432bis], chapter 8.5)

- Extended Default ([RFC8584], chapter 4)

- Highest Random Weight (HRW) ([RFC8584], chapter 3)

- Highest Preference ([Pref-DF])

- Low Preference ([Pref-DF])

#### Algorithm Negotiation

The algorithm configured by command service-carving is advertised in the Ethernet Segment Route (4), DF Election Extended Community, field ‘DF Alg’. If the router receives at least one advertisement without the DF Election Extended Community or with a value of ‘DF Alg’ differing from its advertised value, the router must fall back to the Default Algorithm defined in [RFC7432bis].

### Signaling Primary and Backup DF Elected PEs (EVPNS-REQ#130)

The Signaling Primary and Backup DF Elected PEs (see [RFC7432bis], chapter 8.6) MUST be supported.

### Aliasing Path (EVPNS-REQ#140)

The Aliasing path (see [RFC7432bis], chapter 8.4) MUST be supported.

### Single-Active, All-Active and Load Balancing (EVPNS-REQ#150)

The Single-Active and ALL-Active modes and the Load Balancing of Unicast Packets (see [RFC7432bis], chapter 14) MUST be supported.

### MAC Mobility (EVPNS-REQ#160)

The MAC Mobility (see [RFC7432bis], chapter 15) MUST be supported.

### Fast Convergence (EVPNS-REQ#170)

The Fast Convergence (see [RFC7432bis], chapter 8.2) MUST be supported.

### Split Horizon (EVPNS-REQ#180)

The Split Horizon (see [RFC7432bis], chapter 8.3) MUST be supported.

### Interoperability with Single-Homing PEs (EVPNS-REQ#190)

The multihoming procedures MUST be supported even by single-homing PEs (see [RFC7432bis], chapter 8.7). The document replaces SHOULD to MUST in the second rule into [RFC7432bis], chapters 8.7:

For single-homing PEs, all the above multihoming procedures can be omitted; however, to allow for single-homing PEs to fully interoperate with multihoming PEs, some of the multihoming procedures described above MUST be supported even by single-homing PEs.

## Best Path Selection (EVPNS-REQ#200)

The Best Path Selection algorithm for EVPN routes (see [RFC7432bis], chapter 7.13) must be supported.

## Forwarding

### Forwarding Unicast Packets (EVPNS-REQ#210)

Forwarding Unicast packets (see [RFC7432bis], chapter 13) MUST be supported.

### Multicast and Broadcast (EVPNS-REQ#220)

Only Ingress Replication is supported, P2MP or MP2MP LSPs are not supported.

The document replaces SHOULD to MUST in the second rule in [RFC7432bis], chapters 8.3.1.1:

- An ingress PE (DF or Non-DF) MUST include the ESI label distributed by each non-DF egress PE in the copy of a BUM packet sent to it.

## PMSI Tunnel attribute (EVPNS-REQ#230)

The Inclusive Multicast Ethernet Tag Routes must include P-Multicast Service Interface Tunnel (PMSI Tunnel) attribute ([RFC6514], chapter 5). In the case of Ingress Replication, the attribute must contain the following values:

- Flag:			0

- Type:			6 (Ingress Replication)

- MPLS Label:		LT3 label

- Tunnel Identifier:	Originating Route’s IP Address

## L2-Attr Extended Community (EVPNS-REQ#240)

If at least, one received Ethernet A-D Per EVPN Instance Route does not include the L2-Attr Extended Community, then the [RFC7432], chapter 14.1.1 is applied, otherwise [RFC7432bis], chapter 14.1.1 is applied.

## ES-Import Route Target (EVPNS-REQ#250)

The value of the ES-Import Route Target Extended community ([RFC7432bis], chapters 7.6) carried into an EVPN Ethernet Segment Route is set to the high-order 6-octet ESI Value.

|  | Note: This extended community is used for the Ethernet Segment route filtering like the RT extended community for the other EVPN routes ([RFC7432bis], chapter 8.1.1). |
| --- | --- |

# EVPN Routes

The following table summarizes information passed into the EVPN Routes:

Table 5: EVPN Routes

| Route | Per EVI | Keys | Data | Usage in EVPN functions |
| --- | --- | --- | --- | --- |
| Ethernet A-D Per Ethernet Segment Route (1) | Yes | - Type = 1 - RD - ESI - ET ID = MAX-ET | - ESI Label Ext. Comm. - LT4: per <EVI> - Single-Active flag - NH = Advertising PE’s IP Address - RTs | - Fast Convergence - Split-horizon Filtering |
| Ethernet A-D Per EVPN Instance Route (1) | Yes | - Type = 1 - RD - ESI - ET ID | - LT2: per <EVI, ES, ET> - L2-Attr. Ext. Comm. - Primary flag - Backup flag - NH = Advertising PE’s IP Address - RTs | - Aliasing and Backup Path |
| MAC/IP Address Advertisement (2) | Yes | - Type = 2 - RD - ET ID - MAC Address | - LT1: per <EVI, ET, MAC Address> - ESI - MAC Mob. Ext. Comm. - Static Flag - Sequence Number - NH = Advertising PE’s IP Address - RTs | - MAC Learning |
| Inclusive Multicast Ethernet Tag Route (3) | Yes | - Type = 3 - RD - ET ID - Originating Router’s IP Address | - LT3: per <EVI, ET> - L2-Attr. Ext. Comm - L2 MTU - Flow Label flag - Control Word flag - NH = Advertising PE’s IP Address - RTs | - BUM flooding |
| Ethernet Segment Route (4) | No | - Type = 4 - RD - ESI - Originating Router’s IP Address | - DF Election Ext. Comm. - DF Alg - DF Precedence - Pref-DF - AC-DF Capability - ES-Import Ext. Comm. - ES-Import RT - NH = Advertising PE’s IP Address | - ES auto-discovery - DF Election |

The following chapters briefly describe the EVPN Routes: its contents and when they are created.

## Ethernet A-D Per Ethernet Segment Route (1) (EVPNS-REQ#260)

The route contains the following fields:

- RD				Key

- ESI				Key

- ET ID = MAX-ET		Key

- MPLS Label = 0

- ESI Label Extended Community

- Single-Active flag

- ESI Label (for split horizon function), label of type LT4: <EVI, ES>

Note. This label MUST have the same value in each Ethernet A-D per ES route advertised for the ES ([RFC7432bis], chapter 8.2.1).

Proposed treatment of a route with different label value from already saved ones:

- Save the route and don’t use it until all other such routes are either withdrawn or updated to the same value of this label.

- Turn on the corresponding “Mismatched Configuration” alarm. The alarm must be turned off when this mismatch is resolved.

- RTs

- Next Hop field of the MP_REACH_NLRI attribute = IP address of advertising PE

Each EVPN instance must advertise these routes for each of its ES.

## Ethernet A-D Per EVPN Instance Route (1) (EVPNS-REQ#270)

The route contains the following fields:

- RD				Key

- ESI				Key

- ET ID				Key

- MPLS Label LT2: per <EVI, ES, ET>

Note. The value of ET is VLAN-ID in the case of VLAN-aware Bundle service type and 0 in the other cases.

Label LT2 specifies the sub-interface configured on an ES in the case of ac-based or the ES itself in the other cases.

- L2-Attr Extended Community

  - Primary flag

  - Backup flag

- RTs

- Next Hop field of the MP_REACH_NLRI attribute = IP address of advertising PE.

Each EVPN instance must advertise these routes for its ES and each attached to it BD.

Note. The Ethernet A-D per EVI route may be received by a remote PE before it receives the set of Ethernet A-D per ES routes. Therefore, in order to handle corner cases and race conditions, the Ethernet A-D per EVI route MUST NOT be used for traffic forwarding by a remote PE until it also receives the associated set of Ethernet A-D per ED routes ([RFC7432bis], chapter 8.4).

## MAC/IP Address Advertisement (2) (EVPNS-REQ#280)

The route contains the following fields:

- RD				Key

- ESI

- ET ID				Key

- MAC Address			Key

- IP Address			Key

- MPLS Label LT1

- MPLS Label (L3 Label)

- MAC Mobility Extended Community (only if relearning)

  - Static Flag

  - Sequence Number

- RTs

- Next Hop field of the MP_REACH_NLRI attribute = IP address of advertising PE

Each EVPN instance must advertise these routes for each of its locally learned MAC address. The MPLS labels LT1 of MAC addresses assigned by this system and located into the same MACT (learnt into the same VPN) have the same value, but these labels LT1 in the received routes may have different values.

## Inclusive Multicast Ethernet Tag Route (3) (EVPNS-REQ#290)

The route contains the following fields:

- RD					Key

- ET ID					Key

- Originating Router’s IP Address	Key

- PMSI Tunnel attribute

  - Flag = 0

  - Type = 6 (Ingress Replication)

  - MPLS Label, LT3:

  - Tunnel Identifier = Originating Route’s IP Address

- L2-Attr Extended Community

  - L2 MTU

  - Flow Label flag

  - Control Word flag

- RTs

- Next Hop field of the MP_REACH_NLRI attribute = IP address of advertising PE

Each EVPN instance must advertise these routes for each of its Broadcast Domain.

## Ethernet Segment Route (4) (EVPNS-REQ#300)

The route contains the following fields:

- RD					Key

- ESI					Key

- Originating Router’s IP Address	Key

- ES-Import Route Target Extended Community

- Type: 0x06

- Sub-Type: 0x02 (RT)

- ES-Import: the high-order 6-octet portion of the 9-octet ESI value

- DF Election Extended Community ([RFC8584], chapter 2.2):

- DF Alg

- 0: Default DF election algorithm, or modulus-based algorithm ([RFC7432bis] and ([RFC8584]).

- 1: HRW Algorithm ([RFC8584], chapter 3).

- 2: Highest-Preference algorithm [Pref-DF]

- 3: Lowest-Preference algorithm [Pref-DF]

- Types 4-30: Unassigned.

- 31 Reserved for Experimental Use.

- Bitmap (2 octets)

- Bit 0: [Pref-DF] D bit or 'Don't Preempt' bit (DP hereafter), determines if the PE advertising the ES route requests the remote PEs in the ES not to preempt it as DF.  The default value is DP=0, which is compatible with the 'preempt' or 'revertive' behavior in the Default DF Alg [RFC7432bis].  The DP capability is supported by Alg 2 and Alg 3, and MAY be used with DF Alg 0 or 1.  The procedures of the DP capability for DF Alg 0 or 1 are out of the scope of [Pref-DF].

- Bit 1: ([RFC8584]) AC-DF Capability (AC-Influenced DF Election, ([RFC8584], chapter 4)).

- Bits 2-15: Unassigned.

- DF Preference ([Pref-DF]): defines a 2-octet value that indicates the PE preference to become the DF in the ES.  The allowed values are within the range 0-65535, and the default value MUST be 32767.  This value is the midpoint in the allowed Preference range of values, which gives the operator the flexibility of choosing a significant number of values, above or below the default Preference.  The DF Preference field is specific to DF Alg 2 and DF Alg 3 and does not represent any Preference value for other Algs.  If the DF 3 is different than Alg 2 or Alg 3, these two octets can be encoded differently.

- Next Hop field of the MP_REACH_NLRI attribute = IP address of advertising PE

A PE advertises these routes for each its ES.

If the DF Election Extended Community is omitted, the Default DF election algorithm is assumed.

The ES-Import Route Target Extended Community is used instead of the RT Extended Community in the import/export operations.

|  | Note: Since a PE to import the ES routes using the ES-Import Route Target Extended Community, it imports only the ES routes corresponded to its locally configured ESs. |
| --- | --- |

# EVPN Label Types

This document defines the following types of EVPN MPLS labels:

- LT1

- LT2

- LT3

- LT4

The following table summarizes information about EVPN labels:

Table 6: EVPN MPLS Label Types

| Label | Route and Scope | Usage in Functions | Usage in Data Messages |
| --- | --- | --- | --- |
| LT1 | - MAC/IP Address Advertisement Route (2) - RD Key - ESI - ET Key - MAC Address Key - IP Address Key - LT1 Label - RT - Scope: <EVI, ET, MAC> - Maps to MACT |  | - Forwarding Unicast Packets ([RFC7432bis], chapter 13.1). |
| LT2 | - Ethernet A-D Route per EVPN Instance Route (1) - RD Key - ESI Key - ET Key - LT2 Label - RTs - Scope: <EVI, ET, ES> - Maps to - AC-based: sub-interface - Others: LAG | - Unicast Know Forwarding via Aliasing Path ([RFC7432bis], chapter 8.4) | - Forwarding Unicast Known packets via an Alias path ([RFC7432bis], chapters 8.4). |
| LT3 or BUM | - Inclusive Multicast Ethernet Tag Route (3): - RD Key - ET Key - IP Address Key - PMSI Tunnel attribute - L3 Label - RTs - Scope: <EVI, ET> - Maps to - DF: all interfaces of the BD - Non-DF: all interfaces of the BD, except this ES |  | - BUM packets sent by DF or sent to non-DF. |
| LT4 or ESI | - Ethernet A-D Route per Ethernet Segment Route (1) - RD Key - ESI Key - ET=MAX-ET Key - ESI Label Extended Community - LT4 Label - RTs - Scope: <EVI, ES> - Maps to LAG | - Split Horizon ([RFC7432bis], chapter 8.3). - BUM Forwarding from Non-DF to DF. - Backup Path ([RFC7432bis], chapter 8.4) | - BUM packets sent by non-DF to DF |

### LT1 (EVPNS-REQ#310)

Each EVPN instance creates an LT1 label for each of its MACT and distributes it into MAC/IP Address Advertisement Routes ([RFC7432], chapter 7.2).

The label specifies the MACT.

If a known unicast packet is forwarded to a remote PE that has advertised the MAC/IP Address Advertisement route with label LT1, the label LT1 is added to the packet ([RFC7432bis], chapter 8.4).

In an input PE, an LT1 label specifies the MACT used to forward this frame.

### LT2 (EVPNS-REQ#320)

Each EVNP instance creates an LT2 label for each ES and distributes it into Ethernet A-D Route per EVPN Instance Routes ([RFC7432bis], chapter 7.1).

If a known unicast packet is forwarded to a remote PE that has not advertised the MAC/IP Address Advertisement route with label LT1, the LT2 label is added to the packet ([RFC7432bis], chapter 8.4).

In an input PE a LT2 label specifies the outgoing ES and ET (sub-interface if ET != 0).

### LT3 (EVPNS-REQ#330)

Each EVNP instance creates an LT3 label for each BD and distributes it into Inclusive Multicast Ethernet Tag Routes (in the PMSI Tunnel attribute) ([RFC7432bis], chapter 7.3).

The label specifies the EVNP instance’s non-ES interfaces and ES interfaces on which the PE is the DF.

When a PE forwards a BUM message via MPLS it adds the label if the remote PE is not the DF on the input ES.

### LT4 (EVPNS-REQ#340)

Each EVPN instance creates an LT4 label (ESI label) for each its ES and distributes it into Ethernet A-D Route per Ethernet Segment Routes (in ESI Label Extended Community) ([RFC7432bis], chapter 7.1).

When non-DF PE forwards a BUM message to the remote DF PE it adds the LT4 label after the LT3 label.

# EVPN Data Path

## Forwarding Tables

The chapter describes the EVPN Forwarding Tables.

### MAC Unicast Forwarding Table (EVPNS-REQ#350)

Each EVPN instance creates one MAC Unicast Forwarding Table (MACT). The table is used for forwarding known unicast MAC frames with known Destination MAC addresses is selected by

- The input attachment circuit if the MAC frame arrives from a local CE.

- The frame EVPN label (LT1) is if the MAC frame arrives from a remote CE.

Each entry of these tables contains the following fields:

- VLAN-ID			Key

|  | Note: This field must contain 0 if the EVPN Instance service type differs from vlan-aware-bundle. |
| --- | --- |

- MAC Address			Key

- ESI

- Next Hop – one from the following types:

- Local Next Hop, one of the following types:

  - Attachment circuit

    - ifIndex

- Remote Next Hop, one of the following types:

  - Primary (Single-Active)

    - IP Address

    - Label of type LT1

  - List of Backup Paths (Single-Active)

    - IP Address

    - Label of type LT2

  - Balancing Group List (All-Active)

    - IP Address

    - Label

      - Of type LT1, if non-Alias path (the MAC/IP Advertisement Route has been received)

      - Of type LT2, if Alias path (the MAC/IP Advertisement Route has not been received)

The following EVPN Routes are used to manage MACTs:

- MAC/IP Address Advertisement Route (2)

- Ethernet A-D Route per EVPN Instance Route (1)

### BUM Split Horizon Table (EVPNS-REQ#360)

Each EVPN instance creates one BUM Split Horizon Table – one entry for each interface configured on an Ethernet segment.

The entry of the table contains the following fields:

- MPLS Label LT4	Key

- ifIndex

The table is used for forwarding with Split Horizon BUM MAC frames arriving from remote PEs:

The frame is flooded to all local interfaces specified by the LT3 label except interface specified by the LT4 label.

The following EVPN Routes are used for this table management:

- Ethernet A-D Route per Ethernet Segment Route (1)

### BUM Forwarding Table

Each EVPN instance creates two BUM Forwarding Tables: one for input local traffic LBUMT) and one for input remote traffic (RBUMT).

#### LBUMT

- Input ifIndex

- VLAN-ID

The following table specifies the VLAN-ID value:

Table 7: VLAN-ID value

| Service Type | Mapping |
| --- | --- |
| vlan-based | 0 |
| port-based |  |
| vlan-bundle | VLAN-ID from the input frame |
| vlan-aware-bundle |  |

- List of local interfaces

- List of remote PEs

- IP Address

- MPLS Label ET3 (BUM label)

- MPLS Label LT4 (ES label) or 0.

#### RBUMT

- MPLS Label ET3 (BUM label)		Key

- MPLS Label LT4 (ES label) or 0

- List of local interfaces

## Forwarding Rules (EVPNS-REQ#370)

### General Forwarding Rules

- A frame arriving from a local CE is never forwarded back to this local CE.

- A frame arriving from a remote CEs is forwarded only to the local CEs.

- A BUM frame arriving from a remote CEs is never forwarded to an ES if the PE is non-DF PE on this ES.

### Local Input Traffic

- The input interface specifies the EVPN instance and therefore its TMAC Table.

- Unicast Known MAC frames

- Forward using the MACT table.

- BUM MAC frames

- Forward using the BUM Database.

### Remote Input Traffic

- LT1 Label

- Forward using the MACT, specified by this label. If the MACT’s entry does not exist, then DROP.

- LT2 Label

- Forward on the ES, specified by this label.

Note. In the case of ac-based service type this label specifies this sub-interface, in all other cases it specifies this ES (the LAG).

- LT3 Label

- Forward on the list of ACs, specified by this label.

- LT3, LT4 Label

- Forward on the list of ACs, specified by the LT3 label, except the interface (ES) specified by the LT4 label.

# Configuration (EVPNS-REQ#380)

To configure an EVPN instance the following components must be configured:

- LACP for each Ethernet Segment

- VLAN-ID translation if it is required.

- Ethernet Segment

- BGP

- EVPN instance itself.

The following chapters give an example of EVPN configuration of the ac-based service type.

## Interface Configuration

### LACP Configuration

config

interface agg-eth 1

lacp active

lacp-system-mac 00:45:12:00:04:00

lacp-admin-key 233

### Interface and VID-ID Translation Configuration

config

interface agg-eth 1

l2-transport enable

interface agg-eth 1.100

l2-transport enable

interface x-eth 0/0/2

l2-transport enable

interface x-eth 0/0/2.25

l2-transport enable

### Ethernet Segment Configuration

config

interface agg-eth 1

l2-transport enable

ethernet-segment

identifier type 0 1a.2b.3c.4d.5e.6f.77.88.99

load-balancing-mode all-active

service-carving highest-random-weight

## BGP Configuration

config

routing bgp 123

vrf default

router-id 10.10.10.10

neighbor 10.10.10.1

remote-as-number 123

description MPLSFACING-PEER1

af-l2vpn evpn

neighbor 10.10.10.2

remote-as-number 123

description MPLSFACING-PEER2

af-l2vpn evpn

neighbor 10.10.10.3

remote-as-number 123

description MPLSFACING-PEER3

af-l2vpn evpn

## EVPN Configuration

config

l2-services

evpn evpn1

service-type vlan-based

advertise-mac

auto-discovery

export-rt 100:200

import-rt 100:200

mac-limit 500

interface agg-eth 1.100

interface x-eth 0/0/2.25

# Alarm and Syslog Messages

## Alarms (EVPNS-REQ#390)

The following alarms must be supported:

- Multi-homed ES misconfiguration for ESI esi-value(agg-eth n): from ip-address, LACP System MAC MAC-address, DF Algorithm algorithm.

- Duplicated MAC address mac-address is frozen temporarily (see [RFC7432bis], chapter 15.1).

- Duplicated MAC address mac-address is frozen permanently (see [RFC7432bis], chapter 15.1).

- Relearning of static MAC address mac-address from ip-address (see [RFC7432bis], chapter 15.2).

## Syslog Messages (EVPNS-REQ#400)

The following syslog messages must be supported:

- Every time the state if an alarm is changed the corresponded syslog message must be sent with the following severity:

- OFF => ON: warning

- ON => OFF: informational

- TBD

# Implementation

## Development Milestones

See [Phases].

## EVPN Top Level Architecture

The following figure depicts the EVPN Top Level Architecture

BGP/

EVPN

ConfD

`

CPM

NSM/

EVPN

LC

FIB/EVPN

BCMRM

6

EVPN

Handler

pthread

EVPN

Driver

SDK

SDK pthread

MAC Table Change

callback

MAC Table Change

pthread

TIPC or another

inter-process communication

Limited FIFO

Direct Call

Interface

Manager

MAC

Driver

Figure 3: EVPN Top Level Architecture

## BCMRM MAC Learning support

The following figure depicts a part of the BCMRM Top Level Architecture used by MAC learning:

NSM/EVPN

CPM

1, 2, 3

BCMRM

EVPN Handler

pthread

MACT

Change

pthread

SDK

MACT Driver

SDK phread

MAC Change Callback

TIPC

FIFO of fixed size

Calls

1 – local dynamic MAC addresses,

2 – local static MAC addresses,

3 – remote controlled MAC addresses

1

LC

FIB/EVPN

1, 2, 3

Figure 4: BCMRM MAC Learning Top Level Architecture

|  | Note: It is recommended that the MAC Change callback supports a small cache to drop duplicated BCM messages: - If (received message is saved into the cache) Drop the message. - If (there is not a free room into the cache) Delete the oldest message. - Save the received message into the cache. |
| --- | --- |

Only the higher layers are responsible for managing (adding, updating and deleting entries) the MAC Address Tables.

BCMRM together with BCM must only detect the needed cases and send the messages to the CPM without changing the MAC Address tables.

### MAC Address Type

The following MAC address types are differed:

- Local static

These MAC addresses are configured by a user.

- Local dynamic

These MAC addresses are learned from the local traffic.

- Remote controlled

These MAC addresses are received via the EVPN Control plan.

The MAC address type is received by BCMRM into the “MAC Address Add/Update” from a higher level.

### Aging Time

The time aging must be applied only to local dynamic MAC addresses.

### MAC Learning

The MAC addresses are leaned only from local traffic. If a frame is received on an local interface its Source MAC is sent to the CPU in the following cases:

- Uknown

- Learned on another local interface

- Remote controlled.

# Debug CLI

## BGP

(EVPN-REQ#600) The BGP Debug commands must support the EVPM Address Family:

- AFI = 25 (L2VPN)

- SAFI = 70 (EVPN)

## EVPN

The following command must be supported:

- debug evpn event

### debug evpn events

(EVPN-REQ#610) The command must support the following events:

- Multihoming Events

- EVPN Ethernet Segment is created with the following output:

- EVPN Ethernet Segment Identifier

- Attachment circuit

- Configured Carving Service (Default Forwarder Election method)

  - Configured

  - Negotiated

- EVPN Ethernet Segment is removed with the following output:

- EVPN Ethernet Segment Identifier with the following output:

- EVPN Ethernet Segment connected PE is detected with the following output:

- EVPN Ethernet Segment Identifier

- IP Address

- Configured Carving Service (Default Forwarder Election method)

  - Configured

  - Negotiated

- EVPN Instance Events

- EVPN Instance is created with the following output:

- EVPN Instance name

- EVPN Instance is removed with the following output:

- EVPN Instance name

- Ethernet Segment is added with the following output:

- EVPN Instance name

- Ethernet Segment Identifier

- Ethernet Segment is removed with the following output:

- EVPN Instance name

- Ethernet Segment Identifier

- Attachment Circuit is added with the following output:

- EVPN Instance name

- ifIndex

- Attachment Circuit is removed with the following output:

- EVPN Instance name

- ifIndex

- Remote PE is detected with the following output:

- EVPN Instance name

- PE IP address

- Split Horizon Label

- Remote PE is removed with the following output:

- EVPN Instance name

- PE IP address

- Remote EVPN Instance Ethernet Segment is detected with the following output:

- EVPN Instance name

- Ethernet Segment Identifier

- PE IP address

- Label

- Primary/Backup/None

- DF

Values: local or IP address of remote PE

- Remote EVPN Instance Ethernet Segment is removed with the following output:

- EVPN Instance name

- Ethernet Segment Identifier

- PE IP address

- DF

Values: local or IP address of remote PE

- MAC Address Events with the following output:

- Remote MAC Address is installed with the following output:

- EVPN Instance name

- VLAV

- MAC Address

- Ethernet Identifier

- Source BGP Address

- Source BGP MAC Label

- Sequence Number

- Remote MAC Address is removed with the following output:

- EVPN Instance name

- VLAV

- MAC Address

- Reason

Values: withdraw, flap s, or host moves

- Local MAC Address is installed with the following output:

- EVPN Instance name

- VLAV

- MAC Address

- Type

Value dynamic or static

- Attachment circuit

- Ethernet Identifier

- Local MAC Address is removed with the following output:

- EVPN Instance name

- VLAV

- MAC Address

- Reason

Values: aged out, interface was removed or down

# Appendix

## Appendix A – General & SW Abbreviations

Table 8: General and SW Abbreviations

| AAA | Authentication Authorization & Accounting |  | MLD | Multicast Listener Discovery |
| --- | --- | --- | --- | --- |
| ABF | ACL Based Forwarding |  | MO | Managed Object |
| ACE | Access Control Entry |  | MPLS | Multi Protocol Label Switching |
| ACL | Access Control List |  | MRD | Marketing Requirements Document |
| ADJ | Adjacency |  | MSB | Most Significant Bit |
| ANSI | American National Standardization Institute |  | NSM | Network Service Module |
| AP | Access Point |  | OEM | Original Equipment Manufacturer |
| AUX | Auxiliary |  | OS | Operating System |
| BFD | Bidirectional Forwarding Detection |  | OSPF | Open Shortest Path First (IGP protocol) |
| BGP | Border Gateway Protocol |  | PBR | Policy Based Routing |
| CE | Control Element |  | PD | Platform Dependent |
| CLI | Command Line Interface |  | PI | Platform Independent |
| CPM | Centralized Processing Module (route processor card) |  | PIM | Protocol Independent Multicast (family of multicast protocols) |
| CRC | Cyclic Redundancy Check |  | PIM-SM | PIM Sparse Mode (PIM protocol) |
| CTQ | Critical to Quality |  | POP | Point Of Presence |
| CUPL | Compass Unified Policy Language |  | PRD | Product Requirement Document |
| DAL | Distribution Abstraction Layer |  | QoS | Quality of Service |
| DHCP | Dynamic Host Configuration Protocol |  | RAM | Random Access Memory |
| DUT | Device Under Test |  | RIB | Routing Information Base |
| ETRD | External Technical Requirement Document |  | RPC | Remote Procedure Call |
| EWB | Embedded Web |  | RPF | Reverse Path Forwarding |
| FA | Feature Agent (LC part of feature application) |  | RSVP | Resource ReSerVation Protocol |
| FAA | Feature Agent Application (feature specific part of FA) |  | Rx | Received |
| FAI | Feature Agent Infra (common lib of all FAs) |  | S/N | Serial number |
| FCS | First Customer Shipment |  | sACL | security ACL |
| FE | Forwarding Element |  | SAL | SW Abstraction Layer |
| FIB | Forwarding Information Base |  | SDD | Software Design Document |
| FAM | FAult Management |  | SDP | Software Development Plan |
| FM | Feature Manager (CPM part of feature application) |  | SFS | System/SW Functional Specifications |
| FMI | Feature Manager Infra (common lib of all FMs) |  | SHA | Secure Hash Algorithm |
| FMA | Feature Manager Application (feature specific part of FM) |  | SIB | Sub-channel Information Base |
| FRR | Fast ReRoute |  | SNMP | Simple Network Management Protocol |
| FTP | File Transfer Protocol |  | SSDD | System, Sub-system Design Document |
| HA | High Availability |  | SSH | Secure Shell |
| HAL | Hardware Abstraction Layer |  | STP | Software Tests Plan |
| HSI | Hardware to Software Interface |  | STR | Software Tests Report |
| HTTP | Hypertext Transfer Protocol |  | SW | Software |
| IPC | Inter Protocol Communication |  | TBD | To Be Determined |
| IDB | Interface Descriptor Block |  | TCP | Transmission Control Protocol |
| IGMP | Internet Group Management Protocol |  | TE | Traffic Engineering |
| IGP | Interior Gateway Protocol |  | TFTP | Trivial File Transfer Protocol |
| ILM | Incoming Label Map (MLPS table) |  | TIPC | Transparent Inter-Process Communication |
| IPMI | Intelligent Platform Management Interface |  | Tx | Transmit |
| IPRS | Internal Packet Routing System |  | UC | Unicast |
| ISIS | Intermediate System to Intermediate System (IGP protocol) |  | Ucode | microcode |
| LAG | Link Aggregation |  | UDP | User Datagram Protocol |
| LAN | Local Area Network |  | UM | Unified Manager |
| LBL | Load Balancing |  | URPF | Unicast Reverse Path Forwarding |
| LC | Line Card |  | UTP | Unit Test Procedure |
| LDP | Label Distribution Protocol |  | VLAN | Virtual Local Area Network |
| LSB | Least Significant Byte |  | VSA | Vendor-Specific Attributes |
| MAC | Media Access Control |  | WLAN | Wireless Local Area Networks |
| Mbps | Megabits Per Second |  | WMAN | Wireless Metro Area Network |
| MC | Multicast |  | WPAN | Wireless Personal Area Networks |
| MGID | Multicast Group ID |  | WWAN | Wireless Wide Area Networks |
| MIB | Management Information Base |  | XML | eXtensible Markup Language |

End of Document

This page is intentionally left blank.